| COMPARING THE IMPROVED ACCURACY FROM AVERAGING OVER TIME WITH WAAS, PP PSEUDO RANGE, CARRIER PHASE DATA, AND BEST ACCURACY DATA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MULTIPLE NGS MARKS BayPt, GPS23, and Haycadth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| **** GPS V WAAS Average Position **** |  |  |  |  |  | ** GPS V Differential Post Processed data ** |  |  |  |  |  |  | ** GPS V Differential Post Processed data ** |  |  |  |  | ** GPS V Differential Post Processed data ** |  |  |  |  |  |
|  |  |  |  |  |  |  | Pseudo Range Position |  |  |  |  |  | Carrier Phase Position |  |  |  |  | Best Accuracy Position |  |  |  |  |  |
| Averaging Period in Minutes for Each Sample | WAAS AVG | WAAS MEDIAN | WAAS Approx 93.3 to 94.9 % Error | WAAS Max ERROR |  | PP PR AVG | PP PR MEDIAN | PP PR Approx 93.1 to 94.9 % Error | PP PR Max ERROR | PP Ref Data Interval | # of WAAS Samples |  | PP CP AVG | PP CP MEDIAN | PP CP Approx 93.1 to 94.9 % Error | PP CP Max ERROR |  | BEST ACC. AVG | BEST ACC MEDIAN | BEST ACC Approx 93.1 to 94.9 % Error | BEST ACC Max ERROR |  | Averaging Period in Minutes for Each Sample |
| 0 | 1.767 | 1.731 | 2.756 | 4.432 |  |  |  |  |  | 2 | 18384 |  |  |  |  |  |  |  |  |  |  |  | 0 |
| 1 | 1.732 | 1.714 | 2.526 | 3.441 |  | 0.746 | 0.660 | 1.480 | 2.628 | 2 | 611 |  | 2.014 | 0.822 | 7.896 | 21.023 |  | 0.575 | 0.514 | 1.265 | 2.628 |  | 1 |
| 2 | 1.708 | 1.687 | 2.373 | 2.968 |  | 0.678 | 0.630 | 1.400 | 2.102 | 2 | 305 |  | 0.900 | 0.480 | 4.271 | 9.413 |  | 0.455 | 0.405 | 0.936 | 2.102 |  | 2 |
| 4 | 1.691 | 1.676 | 2.254 | 2.535 |  | 0.605 | 0.554 | 1.188 | 1.847 | 2 | 152 |  | 0.521 | 0.328 | 1.468 | 3.339 |  | 0.377 | 0.352 | 0.758 | 0.933 |  | 4 |
| 8 | 1.673 | 1.692 | 2.038 | 2.227 |  | 0.479 | 0.407 | 0.922 | 1.676 | 2 | 76 |  | 0.322 | 0.265 | 0.663 | 1.051 |  | 0.283 | 0.253 | 0.572 | 0.824 |  | 8 |
| 16 | 1.666 | 1.707 | 1.991 | 2.111 |  | 0.453 | 0.376 | 0.934 | 0.987 | 2 | 37 |  | 0.188 | 0.195 | 0.364 | 0.709 |  | 0.239 | 0.242 | 0.473 | 0.483 |  | 16 |
| 32 | 1.690 | 1.722 | 1.875 | 1.973 |  | 0.437 | 0.441 | 0.711 | 0.861 | 2 | 15 |  | 0.191 | 0.196 | 0.329 | 0.365 |  | 0.212 | 0.210 | 0.336 | 0.349 |  | 32 |